# Adding PMOD IP Core block in Vivado

# Zynq design

Prepared by

Binit Kumar Pandit

# Introduction

In this lab we have added PMOD_ACL to the block design and seen the output of the x, y & z axis value in the terminal.

# Create a Vivado Project

- Open Vivado by selecting Start > All Programs > Xilinx Design Tools > Vivado 2015.4 > Vivado 2015.4.

- Click Create New Project to start the wizard. We will see the Create a New Vivado Project dialog box. Click Next.

- Enter Project name and Project location. Click Next.

- In the Project Type form select RTL Project, and click Next.

- Click Boards. Search for Zedboard Zynq Evaluation and Development Kit and Select it. Click Next. Click Finish.

# Add the Digilent Library Repository

- Find the latest release of Digilent's vivado-library repository where the version number matches the version of Vivado being used from the link https://github.com/Digilent/vivado-library/releases. Download the vivado-library-<version>.zip file (NOT one of the source code archives!), then extract this archive in a memorable location. This GitHub repository contains a large number of IP cores intended for use with Digilent boards, including all of Digilent's Pmod IP cores and Pmod interface description.

- Click Project Settings under Project Manager.

Figure 1: Project Settings to Manage IP

- Click IP then open the Repository Manager tab. Click the  Add button and select the vivado-library folder from where the ZIP archive was extracted to. Click OK.

Figure 2: IP Repository Manager

# Add the Pmod to block design

- In the IP Integrator, Click on “Create Block Design”.

Figure 3: Creation of block design

- Click the “Add IP” icon. Find the “zynq7 processing system” IP and double click it.

Figure 4: Adding “zynq7 processing system” IP

- Find the “PmodACL_v1_0” IP and double click it.

Figure 5: Addition of IP

- The block should appear in the block diagram and we should see the message “Designer Assistance available: Run Block Automation & Run Connection Automation”. Click the Run Connection Automation.

Figure 6: Addition of PmodACL_v1_0 and zynq7 processing system

- First, Click on Run Connection Automation, check all the boxes and then OK.

Figure 7: Connection Automation

We will be able to see this connection

Figure 8: Connection between the different IPs.

- Then, Run Block Automation and click OK.

Figure 9: Block Automation

Figure 10: Overall Connection

- Regenerate Layout: Click the  Regenerate Layout button to rearrange our block design.

Figure 11: Regenerated layout

- Right Click by selecting Pmod_Out in the Pmod_ACL IP from the block diagram and click on Make External to have an external pin.

Figure 12: Making External Pin

- Double Click ZYNQ7 Processing System > Clock Configuration > PL Fabric Clocks > Check FCLK_CLK1. Click OK.

Figure 13: Selecting FCLK_CLK1.

- Now, connect FCLK_CLK1 from ZYNQ7 Processing System to ext_spi_clk of Pmod_Acl_0.

Figure 14: Final Block Diagram

- Select  Validate Design. This will check for design and connection errors.

- Wrapper Creation: Right Click design_1.bd , then select Create HDL Wrapper and click OK.

Figure 15 (a): Creation of HDL Wrapper

Figure 15 (b): Auto-updating of HDL Wrapper

- Generate Bitstream: In the Program and Debug, select Generate Bitstream. And wait till the generation of bitstream.

Figure 16: Bitstream Generation

# Export the hardware design to SDK

Once the bitstream has been generated, we can export our design to SDK where we can then write code for the PS. The PS is going to write data to our multiplier and read back the result.

- In Vivado, from the File menu, select “Export > Export hardware”.

Figure 17: Export to Hardware

- In the window that appears, tick “Include bitstream” and click “OK”.

Figure 18: Bitstream Inclusion

- Again from the File menu, select “Launch SDK”. In the window that appears, use the following settings and click “OK”.

At this point, the SDK loads and a hardware platform specification will be created for our design. We should be able to see the hardware specification in the Project Explorer of SDK as shown in the image below.

Figure 19: Launch SDK

# Create a Software application

At this point, our SDK window should look somewhat like this:

Figure 20: SDK Window

- From the File menu, select New > Application Project.

Figure 21: Selection of Application Project

- In the first page of the New Project wizard, choose a name for the application. We’ve chosen “Pmod_Acl_test”. Click “Next”.

Figure 22: Creation of Application Project

- On the templates page, select the “Empty Application” template and click “Finish”.

Figure 23: Selection of a template

- The SDK will generate a new application which we should find in the Project Explorer as in the image below. From Project Explorer Expand design_1_wrapper_hw_platform_0 > drivers > PmodACL_v1_0 > examples > main.c. Select and copy (Ctrl +  C) main.c and Paste (Ctrl +  V) at Pmod_Acl_test > src.

Figure 24: Source folder of Pmod_ACL_test

# Test the design on the hardware

To test the design, we are using ZedBoard from Avnet.

- In the SDK, from the menu, select Xilinx Tools > Program FPGA.

Figure 25: Selecting Program FPGA

- In the Program FPGA window, we select the hardware platform to program. We have only one hardware platform, so click “Program”.

Figure 26: Programming of FPGA

- Adding terminal: Add terminal to view the generated output. Select Window > Show View > Other > Terminal > Terminal.

Figure 27: Addition of Terminal

- Connecting Terminal

Figure 28: Connecting Terminal

Made the following changes in order to see the output in the terminal:

- Connection Type: Serial

- Port: COM4 (Nt.: It may be different with respect to PC, try selecting  UART to USB Driver).

- Baud Rate: 11520.

Figure 29: Configuration to Terminal

- The bitstream will be loaded onto the Zynq and we are ready to load the software application. Select the “multiplier_test” folder in the Project Explorer, and then from the menu, select Run->Run.

- Click Run As  button, select “Launch on Hardware (System Debugger)” and click “OK”.

Figure 30: Launching Hardware

- Output: Then again click Run to get the output in the terminal.

Figure 31: Output in the Terminal

- We can also see the output in the Tera Term Software.

- Open Tera Term.

Figure 32: Tera Term

- Select Serial > COM4

Figure 33: Selection of COM4: USB Serial Port

- Click Setup > Serial Port. Change Speed to 115200 (Same as Baud Rate).

Figure 34: Setting up of Tera term

- Output can be seen as the given figure

Figure 35: Output in the Tera Term

Figure 36: FPGA dumped with Program

# Creating whole model as IP

We can convert the whole design as a single IP and use the same in any other design.

- Select Tools > Create and Package IP > Package your current project. Click Next.

Figure 37: Packaging the Design

- Click Next.

- Click Finish.

Figure 38: Creating IP from the design.

- Then, follow the same processes as discussed earlier in the creation of a custom IP.